Statement of Work

SONC CIO Conference Room

General:

The requirement is to implement VTC and audiovisual system upgrades in the SONC Conference Room. This system design is based upon many of the other videoconference rooms within the SOCOM facility and utilizes some of the Government Furnished Equipment (GFE) in use in the SONC Conference Room.

Work to specifically include:

One existing Tandberg 6000 MXP CODEC, an additional 6000 MXP CODEC, Crestron control

equipment, AV matrix switcher, audio amplifier and the existing VHS/DVD player will be

integrated into the Government’s rack desk/workstation. The rack will be equipped with a UPS to maintain power to the control equipment improving reliability. Two plasma monitors will be mounted side-by-side flat to front wall for use during dual-monitor videoconference sessions, and the existing projector and screen will remain for use during briefings requiring only a single display device. The projection screen will be moved forward to provide clearance for the new plasma monitors. Sources which will display images on the screen include the CPU located on the desk, a laptop connected to a surface interface on the rack and wall, and the VCR/DVD player. The videoconference CODECs with HD cameras will display on the two plasma monitors. Material originating from any of the computers may be pushed through the videoconference CODECs to the far site. Also, a single microphone connected to a wall plate in the front of the room to be placed on the table will also serve the two CODECs. Audio for the source in use will be heard through the existing speakers. The control system, equipped with a 12” diagonal color touch panel, will be programmed to operate the new system, including system power, source and display selection, VTC control, VCR/DVD control, and audio volume. During videoconference sessions, the system will interrupt power to the unused videoconference CODEC not in use.  The Control system must be capable of integrating into the AV/VTC control LAN being installed in the building.  Crestron RoomView and X-panel are used to remotely access conference room equipment and control said conference rooms.  The look and operation of the touch panel must be similar to all other conference rooms in the building.

SYSTEM IMPLEMENTATION

The Vendor will provide a turnkey multimedia system capable of switching and displaying up to 1080p resolution to include equipment, implementation and warranty. The Vendor Installation includes engineering, coordination and labor for display, video, audio, smart control systems and related equipment to include required plates, connectors and cables.

System Engineering, including:

- Preparation of system functional interconnection diagram.

- Facility and equipment location drawing.

- Project Coordination, including meetings with the client’s technical representatives and

project coordination, including:

- System implementation monitoring.

- Schedule project and oversee the  Vendor’s team

- Equipment staging at our shop

- Assure final punch-list items are completed

Field Labor, including:

- Pulling and bundling cables

- Terminating of all cable

- Labeling all cable

- Mounting and terminating computer interfaces

- Installing structural systems for multimedia equipment

- Program control System

- Adjusting and balancing audio settings

- Assure that the installed systems functions as proposed

- Site cleanup and rubbish disposal

- Provide end-user training

OTHER:

- All equipment, wire and accessories required for a fully functional audio and visual

system.

- Labor associated with turnkey engineering, installation, programming, testing

and training.

- Documentation package including as-built system CAD diagrams and Operation &

Maintenance manuals.

- Ninety day (90 days) Vendor warranty coverage of all materials and craftsmanship.

- Vendor warranty coverage of all materials and craftsmanship.

- End User training on system operation.

- Any additional trips, labor or materials due to failure of the other work forces to have the audiovisual system rough-in work completed as anticipated and previously confirmed, will be added to the project billing as required.

- Where applicable, the owner’s architect will provide The Vendor’s engineering department with all required architectural floor, reflected ceiling, building elevation, and section plans in Auto Cad format or another acceptable format.

- Software licensing – Where applicable, The Vendor will provide a copy of the source code, on CD or DVD, written specifically for the operation of the control systems. This will be provided under a licensing agreement limiting the duplication and distribution of software written by The Vendor for the specific use within the systems installed by The Vendor.

- The Vendor quotation should be based on an installation site in the Tampa, FL area.

Statement of Work

JMISC Conference Room

General:

The requirement is to implement VTC and audiovisual system upgrades in the JMISC Conference Room. This system design is based upon many of the other videoconference rooms within the SOCOM facility and utilizes some of the Government Furnished Equipment (GFE) in use in the JMISC Conference Room.

Work to specifically include:

Replace the existing Elmo camera with an HD camera that will be integrated into the existing Tandberg 6000 MXP CODEC. Install a wall plate for connection of the control touch panel in the front of the room. Improve the workstation layout to provide more space for table occupants and the operator of the workstation. Improve PC audio functions to the desktop and for presenters by adding a switching system to route audio from the desired PC to the room’s audio system and desktop PC speakers.  Upgrade the existing Crestron control system to accommodate new equipment and network access (for RoomView and X-panel).  The look and operation of the touch panel must be reprogrammed to provide the same look and operation as all other conference rooms in the Headquarters.

SYSTEM IMPLEMENTATION

The Vendor will provide a turnkey multimedia system to include equipment, implementation and warranty. The Vendor Installation includes engineering, coordination and labor for any display, video, audio, smart control systems and related equipment to include required plates, connectors and cables.

System Engineering, including:

- Preparation of system functional interconnection diagram.

- Facility and equipment location drawing.

- Project Coordination, including meetings with the client’s technical representatives and

project coordination, including:

- System implementation monitoring.

- Schedule project and oversee the  Vendor’s team

- Equipment staging at our shop

- Assure final punch-list items are completed

Field Labor, including:

- Pulling and bundling cables

- Terminating of all cable

- Labeling all cable

- Mounting and terminating computer interfaces

- Installing structural systems for multimedia equipment

- Program control System

- Adjusting and balancing audio settings

- Assure that the installed systems functions as proposed

- Site cleanup and rubbish disposal

- Provide end-user training

OTHER:

- All equipment, wire and accessories required for a fully functional audio and visual

system.

- Labor associated with turnkey engineering, installation, programming, testing

and training.

- Documentation package including as-built system CAD diagrams and Operation &

Maintenance manuals.

- Ninety day (90 days) Vendor warranty coverage of all materials and craftsmanship.

- Vendor warranty coverage of all materials and craftsmanship.

- End User training on system operation.

- Any additional trips, labor or materials due to failure of the other work forces to have the audiovisual system rough-in work completed as anticipated and previously confirmed, will be added to the project billing as required.

- Where applicable, the owner’s architect will provide The Vendor’s engineering department with all required architectural floor, reflected ceiling, building elevation, and section plans in Auto Cad format or another acceptable format.

- Software licensing – Where applicable, The Vendor will provide a copy of the source code, on CD or DVD, written specifically for the operation of the control systems. This will be provided under a licensing agreement limiting the duplication and distribution of software written by The Vendor for the specific use within the systems installed by The Vendor.

- The Vendor quotation should be based on an installation site in the Tampa, FL area.

Statement of Work

Building 506E 2nd Floor Conference Room

General:

The requirement is to implement VTC and audiovisual system upgrades in the Building 506E 2nd Floor Conference Room. This system design is based upon many of the other videoconference rooms within the SOCOM facility and utilizes some of the Government Furnished Equipment (GFE) in use in the Building 506E 2nd Floor Conference Room.

Work to specifically include:

Replace the existing Tandberg VTC system with a new 6000 MXP CODEC with HD camera. Replace the existing AMX control system with a Crestron standard to include installing a wall plate for the connection of the control touch panel at the front of the room.  The Control system must be capable of integrating into the AV/VTC control LAN being installed in the building.  Crestron RoomView and X-panel are used to remotely access conference room equipment and control said equipment.  The look and operation of the touch panel must be similar to all other conference rooms in the Headquarters.  Improve the workstation layout by relocating it to the rear of the room and installing adjacent to it a new equipment rack containing the AV equipment. Lastly, install new and furnished GFE in a new rack with castors to include a rear connection panel to which the rooms A/V cabling may be connected and disconnected with ease.

SYSTEM IMPLEMENTATION

The Vendor will provide a turnkey multimedia system to include equipment, implementation and

warranty. The Vendor Installation includes engineering, coordination and labor for any display, video, audio, smart control systems and related equipment to include required plates, connectors

and cables.

System Engineering, including:

- Preparation of system functional interconnection diagram.

- Facility and equipment location drawing.

- Project Coordination, including meetings with the client’s technical representatives and

project coordination, including:

- System implementation monitoring.

- Schedule project and oversee the  Vendor’s team

- Equipment staging at our shop

- Assure final punch-list items are completed

Field Labor, including:

- Pulling and bundling cables

- Terminating of all cable

- Labeling all cable

- Mounting and terminating computer interfaces

- Installing structural systems for multimedia equipment

- Program control System

- Adjusting and balancing audio settings

- Assure that the installed systems functions as proposed

- Site cleanup and rubbish disposal

- Provide end-user training

OTHER:

- All equipment, wire and accessories required for a fully functional audio and visual

system.

- Labor associated with turnkey engineering, installation, programming, testing

and training.

- Documentation package including as-built system CAD diagrams and Operation &

Maintenance manuals.

- Ninety day (90 days) Vendor warranty coverage of all materials and craftsmanship.

- Vendor warranty coverage of all materials and craftsmanship.

- End User training on system operation.

- Any additional trips, labor or materials due to failure of the other work forces to have the audiovisual system rough-in work completed as anticipated and previously confirmed, will be added to the project billing as required.

- Where applicable, the owner’s architect will provide The Vendor’s engineering department with all required architectural floor, reflected ceiling, building elevation, and section plans in Auto Cad format or another acceptable format.

- Software licensing – Where applicable, The Vendor will provide a copy of the source code, on CD or DVD, written specifically for the operation of the control systems. This will be provided under a licensing agreement limiting the duplication and distribution of software written by The Vendor for the specific use within the systems installed by The Vendor.

- The Vendor quotation should be based on an installation site in the Tampa, FL area.