|  |  | CAMERA SYSTEMS, HD-SD |
| --- | --- | --- |
| 1 | Lot | Camera Systems, HD/SD for the Educational Communications Center $_____________ (ECC), Kansas State University, as described below: |  |
|  |  | Required Components (Qty 2 each): |
|  |  | 1. Camera Control Unit (CCU) - Sony HXCU-100 or approved equivalent |
|  |  | 2. Camera Head - Sony HXC-100 or approved equivalent |
|  |  | 3. Remote Control Panel - Sony RCP-700/900-series, including Inter- connection cable (e.g. Sony CCA-5), or approved equivalent |
|  |  | 4. Camera Lens - Fujinon ZA17X7.6BERM or Canon KJ17ex7.7B ENG-Type lens or approved equivalent |
|  |  | 5. Viewfinder - Small format (1.5-2.0 inch diagonal) monochrome HD viewfinder (for hand-held operation) - Sony HDVF-20A or approved equivalent |
|  |  | 6. Additional Camera Accessories - to include microphone holder, tripod adapter, and return video selector - Sony CAC-12, VCT-U14, and CAC-6 or approved equivalent(s) |
|  |  | 7. 24/7 Technical support |
|  |  | 8. Shipping |
|  |  | 9. Warranty: Parts______________ Labor ________________ |
|  |  | Optional components/services to include: |
|  |  | 1. Qty 2: Camera Lens Controls - Remote manual focus, remote servo $___________$_____________ zoom, including all necessary clamps, adapters, cables, and handles appropriate for supplied make/model of lens |  |
|  |  | 2. Qty 2: Monochrome Viewfinder - Large format (5 inch diagonal or $__________ $____________ greater) CRT monochrome HD viewfinder for studio operation – Sony HDVF-550 or approved equivalent |  |
|  |  | 3. Qty 2: Color Viewfinder - Large format (5 inch diagonal or greater) $__________ $_____________ LCD color HD viewfinder for studio operation - Sony HDVF-C730W or approved equivalent |  |
|  |  | 4. Qty 2: Hard-shell transport case for camera head, lens, and small $___________$____________ viewfinder |  |
|  |  | 5. Qty 1: Model RMB-150 Remote Control Unit (RCU) for use with $___________$____________ existing Sony PDW-700 Camera |  |
|  |  | 6. Extended Warranty Coverage: Parts__________ Labor___________ $___________$____________ |  |
|  |  | Criteria for purchase consideration: |
|  |  | 1. Reliable operation, format and operational flexibility, with support for current and evolving video and audio standards. Hardware specified in this RFQ must output SD video signals that meet SMPTE 259M digital signal standards, with the capability to also output video signals in full compliance with SMPTE 292M HD digital signal standards. Capable of full 16:9 aspect ratio operation. |
|  |  | 2. Ability to interface with existing analog, SDI, and HD/SDI systems utilizing existing triax cable for SDI and HD/SDI operation at distances up to 1200 meters*. Capable of carrying all necessary video, audio, intercom, and data signals bi-directionally on a single triax cable per camera. Via triax, provide sufficient voltage and current to safely and reliably provide power to an attached camera head, lens, lens controls, and viewfinder. |
|  |  | 3. Ability to accept synchronization signals allowing multiple camera synchronization for multi-camera video production. |
|  |  | 4. Provide multiple return signals to the camera head, including teleprompter, return video (VBS, SD or HD/SDI) tally and intercom. |
|  |  | 5. Optional purchase of appropriate accessories to allow use in a studio environment, including additional large format viewfinder, lens controls (zoom/focus) and remote control unit (RCU) allowing camera shading and color adjustments (X2). |
|  |  | 6. 24/7 technical support, maintenance and warranty service provided by the equipment manufacturer(s) and/or vendor. |
|  |  | General specifications/requirements: |
|  |  | 1. All costs involved in preparation of the quote are the responsibility of the vendor. |
|  |  | 2. All provided hardware is required to meet SMPTE 259M and SMPTE 292M HD standards, depending on acquisition format selected at the time of production. Up-conversion from SD to HD is not acceptable, and all down-conversion is to take place within provided equipment, without external down-converter hardware. |
|  |  | 3. All operation manuals, service manuals, and extender cards (if applicable) are to be provided with the supplied equipment. Operation and service manuals may be provided in digital format, but paper versions are preferred. |
|  |  | 4. NOTE: The ECC will consider current model demonstration or "B-stock" equipment in this request if the condition of the equip-ment is clearly declared, deemed acceptable to the ECC, and all hardware and software provided with the full manufacturer's new-product warranty meeting the warranty requirement listed below. |
|  |  | Demonstration: The Educational Communications Center at Kansas State University reserves the right to refuse any or all bids, and may require an on-site demonstration of any equipment listed in the quote before purchase approval. All costs involved in providing an on-site demonstration are the responsibility of the vendor. |
|  |  | Specifications - Required Components/Services: |
|  |  | 1. Camera Control Unit (CCU) - Sony HXCU-100 or approved equivalent, providing: |
|  |  | Digital triax camera control unit with Kings (or compatible) triaxial connector, providing: |
|  |  | To camera head (from CCU): |
|  |  | Power for camera head and attached peripherals, including viewfinder and ENG/EFP lens and controls, |
|  |  | Intercom audio (camera person earphone), |
|  |  | Tally signals (red and green), |
|  |  | Auxiliary audio (return audio), |
|  |  | HD-SDI or SDI return video, |
|  |  | VBS (NTSC) return video, and/or VBS prompter video, |
|  |  | Reference video (HD (SMPTE 274M), or SD (NTSC Black Burst)), |
|  |  | Remote control data interface. |
|  |  | From camera head (to CCU): |
|  |  | Intercom audio (camera person microphone), |
|  |  | At least two channels of full-bandwidth program audio, |
|  |  | HD/SDI or SD/SDI camera video, |
|  |  | HD/SDI or SD/SDI monitor output, |
|  |  | HD or SD component YUV video and/or RGB video, |
|  |  | VBS out, |
|  |  | Pix out, |
|  |  | Remote control data interface. |
|  |  | CCU must be capable of powering the attached camera, viewfinder, ENG/EFP lens and lens controls a minimum distance of 1000 m (3280 ft) on existing triax cable already in place at Bill Snyder Family Stadium and Bramlage Coliseum on the K-State campus. On-site testing may be required before final acceptance. |
|  |  | Appropriate camera controls (IRIS, PEDESTAL, ATW, ABB, etc.) may be included on the CCU itself, but provision for a remote control unit (RCU) is required to allow operator control at a separate location from the CCU. (See requirement 3 following.) |
|  |  | 2. Camera Head - Sony HXC-100 or approved equivalent, providing: |
|  |  | Digital triax camera head with Kings (or compatible) triaxial connector, providing: |
|  |  | Picture quality, color, and resolution identical to, or better than, that provided by the existing Sony PDW-700 EFP/ENG cameras already in use by the ECC. It is a requirement of this procurement that the video signal output of any cameras provided be indistinguishable (as defined by the ECC) from the video output of the existing PDW-700 cameras, allowing all cameras to be used in productions where more than two cameras are required, Internal camera body filter wheel with clear, 1/4 neutral density (ND), 1/16 ND, and 1/64 ND filters user selectable, Bayonet mount lens mount, supporting 2/3" HD lenses 3-chip 2/3" minimum CCD imaging device with effective picture elements of at least 1920 (H) by 1080 (V), 14-bit or greater analog to digital conversion, HD and SD signal format support including 1080/59.94i, 720/29.97P, 480/59.94i, Sensitivity of at least F10 (59.94 Hz) at 2000 lx, 3200 K, 89.9% reflectance, Signal to noise (S/N) ratio: HD min -55 dB (1080i), SD min -65 dB at 59.94 Hz, Horizontal resolution: HD min 1000 TV lines, SD min 900 TV lines, Shutter capability: 1/100, 1/125, 1/500, 1/1000, 1/2000 (s) in 59.94i mode, Modulation depth: HD 45% at 27.5 MHz (1080i), SD 90% at 5 MHz. |
|  |  | Camera Head I/O: |
|  |  | Audio input |
|  |  | Return control input |
|  |  | Prompter output/genlock input/return video input (depending on presence of CCU) |
|  |  | DC input (4-pin XLR) |
|  |  | DC output (10.5 to 17 VDC at 0.5A max (for wireless microphone, etc.) |
|  |  | Test output |
|  |  | SDI output |
|  |  | Earphone output |
|  |  | Intercom I/O connector to allow direct connection of headset to camera head without external intercom interface (box) |
|  |  | Remote connector |
|  |  | Lens connector (providing power and iris from camera head) |
|  |  | Viewfinder connector (to power attached viewfinder). |
|  |  | Signal conveyance: |
|  |  | From camera head (to CCU): |
|  |  | Intercom audio (camera person microphone), |
|  |  | At least two channels of full-bandwidth program audio, |
|  |  | HD/SDI or SD/SDI digital camera video, |
|  |  | HD/SDI or SD/SDI monitor video, |
|  |  | HD or SD component YUV video and/or RGB video (may be converted from digital video signal at CCU), |
|  |  | VBS out (may be converted from digital video signal at CCU), |
|  |  | Pix out, providing camera menus to remote operator. |
|  |  | To camera (from CCU): |
|  |  | Power for camera head and attached peripherals, including viewfinder and ENG/EFP lens and controls, |
|  |  | Intercom audio (camera person earphone), |
|  |  | Tally signals (red and green), |
|  |  | Program audio (return audio), |
|  |  | HD-SDI or SDI return video, |
|  |  | VBS (NTSC) return video, |
|  |  | Prompter video (VBS), |
|  |  | Reference video (HD (SMPTE 274M), or SD (NTSC Black Burst)), |
|  |  | Remote control data to/from camera head. |
|  |  | 3. Remote Control Panel - Sony RCP-700/900-series, including interconnection cable (e.g. Sony CCA-5), or approved equivalent, providing: |
|  |  | Full control of all standard camera and lens functions via remote panel providing: |
|  |  | "Joystick" iris/pedestal control, |
|  |  | Tally indication, |
|  |  | Ability to adjust and configure camera for appropriate HD and SD standards, |
|  |  | Ability to operate up to 30 meters (100 feet) from CCU on a single interface cable (provided by vendor). |
|  |  | 4. Camera Lens - Fujinon ZA17X7.6BERM or Canon KJ17ex7.7B ENG-Type lens or approved equivalent, providing: |
|  |  | 2X extender, |
|  |  | Sun shield/lens hood, |
|  |  | Clear protective filter, |
|  |  | Automatic or manual iris control, |
|  |  | Servo and manual zoom capability, with provision for remote servo zoom |
|  |  | control, |
|  |  | Manual focus capability, with provision for cabled focus control, |
|  |  | Power requirements within limits provided by the camera, and appropriate mating connector. |
|  |  | 5. Viewfinder - Small format (1.5-2.0 inch diagonal) monochrome HD viewfinder (for hand-held operation) - Sony HDVF-20A or approved equivalent, including: |
|  |  | Appropriate mount and interface connector to allow direct connection of viewfinder to camera body with signal conveyance via viewfinder port, without adapters, |
|  |  | Ability to enable or disable tally signal from camera location, |
|  |  | Ability to view return video from camera CCU when required, |
|  |  | Protective bar over front operator controls. |
|  |  | NOTE: The model HDVF-20A viewfinder is preferred over the model |
|  |  | HDVF-200 viewfinder due to protective features and ECC preference. |
|  |  | 6. Additional Camera Accessories - to include microphone holder, tripod adapter, and return video selector - Sony CAC-12, VCT-U14, and CAC-6 or approved equivalent(s): |
|  |  | Complete set of camera accessories listed above for each camera head. |
|  |  | 7. 24//7 Technical support providing: |
|  |  | Telephone (preferred) or e-mail (acceptable) support of provided camera equipment, available 24/7, 365 days per year, for a period of at least one year from date of purchase. |
|  |  | All relevant software/firmware updates provided at no charge for a period of at least one year from date of purchase. |
|  |  | Specifications - Optional components/requirements: |
|  |  | 1. Camera Lens Controls - Remote manual focus, remote servo zoom, including all necessary clamps, adapters, cables, and handles appropriate for supplied make/model of lens, providing: |
|  |  | Manual focus handle, handle mount, cable of sufficient length, and lens focus block to allow use of camera in a "studio" configuration whereby the operator can control lens focus remotely while standing upright be-hind the camera mounted on a tripod or pedestal. |
|  |  | Servo zoom handle, handle mount, cable, and other required accessories to allow use of camera in "studio" configuration, whereby the operator can control lens zoom remotely while standing upright behind the camera mounted on a tripod or pedestal. |
|  |  | Ability to vary zoom speed using a single digit (thumb or finger). |
|  |  | Zoom handle "return video" control allowing operator to switch view of viewfinder video from camera video to CCU return video (for image composition, etc.). |
|  |  | Lens controls and accessories provided by lens manufacturer preferred. Black or dark accessory color preferred. |
|  |  | 2. Monochrome Viewfinder - Large format (5 inch diagonal or greater) CRT monochrome HD viewfinder for studio operation - Sony HDVF-550 or approved equivalent providing: |
|  |  | Appropriate mount and interface connector to allow direct connection of viewfinder to camera body and signal conveyance via viewfinder port, without adapters, |
|  |  | Ability to view return video from camera CCU when required, |
|  |  | Ability to enable or disable tally signal from camera location. |
|  |  | 3. Color Viewfinder - Large format (5 inch diagonal or greater) LCD color HD viewfinder for studio operation - Sony HDVF-C730W or approved equivalent providing: |
|  |  | Appropriate mount and interface connector to allow direct connection of viewfinder to camera body and signal conveyance via viewfinder port, without adapters, |
|  |  | Ability to view return video from camera CCU when required, |
|  |  | Ability to enable or disable tally signal from camera location. |
|  |  | 4. Hard-shell transport case for camera head, lens, and small viewfinder: |
|  |  | Hard shipping case to allow storage and/or airline transport of provided camera head, attached lens, viewfinder, and relevant camera accessories. |
|  |  | 5. Model RMB-150 Remote Control Unit (RCU) for use with existing Sony PDW-700 Camera: |
|  |  | The ECC currently has two Sony PDW-700 camera systems. A new or acceptable used/"B-stock" Sony RMB-150 RCU for use with the PDW-700, via direct connection of the RMB-150 to the camera head REMOTE connector. This option is to include the necessary interface cable(s) to allow control of the PDW-700 at a minimum distance of 15 meters (approx. 30 feet). |
|  |  | If a Sony Model RMB-150 cannot be provided, the current Sony RCU specified for direct attachment to the PDW-700 camcorder (and appropriate cable) may be specified. |
|  |  | 6. Extended Warranty Coverage (if available): |
|  |  | Additional years, in one-to-three year increments of factory- or vendor-supplied extended warranty beyond the first year. |
|  |  | Ship to the following address: |
|  |  | Educational Communications Center |
|  |  | Kansas State University |
|  |  | 128 Dole Hall |
|  |  | Manhattan, KS 66506 |
|  |  | Warranty: Factory (or manufacturer-approved agent) repair or replace-ment of defective hardware/software for a minimum period of one (1) year (from date of shipment to purchaser) at no additional cost to the purchaser, including freight charges. All provided components (including power supplies) are to be covered by this warranty. |
|  |  | PRICES ABOVE SHALL INCLUDE SHIPPING AND HANDLING CHARGES. |
|  |  | Shipping and F.O.B. Point: Unless otherwise specified, unit prices shall be F.O.B. DESTINATION, PREPAID AND ALLOWED (included in the unit price) delivered to the agency's receiving dock or other designated point as specified in this Request without additional charges. |
|  |  | PLEASE EXTEND PRICES WHEN PREPARING YOUR BID. VENDOR'S PRICING RESPONSE SHALL BE PROVIDED ON THIS BID FORM. Failure to provide information as requested may result in the determination of vendor's response being incomplete and non-responsive. |
|  |  | Questions/Addenda: No pre-bid conference is scheduled for this Request for Quotation. Questions requesting clarification of the RFQ must be submitted electronically (MS Word) to the Procurement Officer at greg.davis@da.ks.gov by 10:00 AM (CST), March 16, 2010. |
|  |  | Failure to notify the Procurement Officer of any conflicts or ambiguities in this RFQ may result in items being resolved in the best interest of the State. Any modification to this Request shall be made in writing by addendum and posted on the Division of Purchases website, http://da.ks.gov/purch. Only written communications are binding. |
|  |  | Electronic bids will not be accepted. |
|  |  | It shall be the responsibility of all participating vendors to acquire any and all addenda and additional information as it is made available from the website cited above. Vendors are required to check the website periodically for any additional information or instructions. |
|  |  | Technical Literature: All bids shall include specifications and technical literature sufficient to allow the State to determine that the equipment meets all requirements. This technical literature will be the primary source for bid evaluation. If a requirement is not addressed in the technical literature it must be supported by additional documentation and included with the bid. Bid responses without sufficient technical documentation may be rejected. |
|  |  | Accounts Receivable Set-Off Program: During the course of this contract if the vendor is found to owe a debt to the State of Kansas, agency pay-ments to the vendor may be intercepted / setoff by the State of Kansas Accounts Receivable Setoff Program. Notice of the setoff action will be provided to the vendor. The vendor shall credit the account of the agency making the payment in an amount equal to the funds intercepted. |
|  |  | K.S.A. 75-6201 et seq. allows the Director of Accounts & Reports to set-off funds the State of Kansas owes vendors against debts owed by the vendors to the State of Kansas.Payments setoff in this manner constitute lawful payment for services or goods received. The vendor benefits fully from the payment because its' obligation to the State is reduced by the amount subject to setoff. |
|  |  | CRIMINAL OR CIVIL OFFENSE OF AN INDIVIDUAL OR ENTITY THAT CONTROLS A COMPANY OR ORGANIZATION OR WILL PERFORM WORK UNDER THIS CONTRACT: Any conviction for a criminal or civil offense that indicates a lack of business integrity or business honesty must be disclosed. This includes (1) conviction of a criminal offense as an incident to obtaining or attempting to obtain a public or private contract or subcontract or in the performance of such contract or subcontract; (2) conviction under state or federal statutes of embezzlement, theft, forgery, bribery, falsification or destruction of records, receiving stolen property; (3) conviction under state or federal antitrust statutes; and (4) any other offense to be so serious and compelling as to affect responsibility as a state contractor. For the purpose of this section, an individual or entity shall be presumed to have control of a company or organization if the individual or entity directly or indirectly, or acting in concert with one or more individuals or entities, owns or controls 25 percent or more of its equity, or otherwise controls its management or policies. Failure to disclose an offense may result in disqualification of the bid or termination of the contract. |

Request for Quotation No.: 12882

Page 10 of 13

Tax Clearance INSTRUCTIONS

PLEASE NOTE:  This information has changed effective 10/27/09

A “Tax Clearance” is a comprehensive tax account review to determine and ensure that the account is compliant with all primary Kansas Tax Laws administered by the Kansas Department of Revenue (KDOR) Director of Taxation.  Information pertaining to a Tax Clearance is subject to change(s), which may arise as a result of a State Tax Audit, Federal Revenue Agent Report, or other lawful adjustment(s).

To obtain a Tax Clearance Certificate, you must:

- Go to http://www.ksrevenue.org/taxclearance.htm to request a Tax Clearance Certificate

- Return to the website the following working day to see if KDOR will issue the certificate

- If issued an official certificate, print it and attach it to your bid response

- If denied a certificate, engage KDOR in a discussion about why a certificate wasn’t issued

Bidders (and their subcontractors) are expected to submit a current Tax Clearance Certificate with every event response.

Per KSA 75-3740-(c), the Director of Purchases may reject the bid of any bidder who is in arrears on taxes due the State of Kansas.  The Secretary of the Kansas Department of Revenue is authorized to exchange such information with the Director of Purchases as is necessary to determine a bidder’s tax clearance status, notwithstanding any other provision of law prohibiting disclosure of the contents of taxpayer records or information.

Please Note:  Individual and business applications are available.  For applications entered prior to 5:00 PM Monday through Friday, results typically will be available the following business day.  Tax clearance requests may be denied if the request includes incomplete or incorrect information.

Please Note:  You will need to sign back into the KDOR website to view and print the official tax clearance certificate.

Information about Tax Registration can be found at the following website:

http://www.ksrevenue.org/busregistration.htm

The Division of Purchases reserves the right to confirm tax status of all potential contractors and subcontractors prior to the release of a purchase order or contract award.

In the event that a current tax certificate is unavailable, the Division of Purchases reserves the right to notify a bidder (one that has submitted a timely event response) that they have to provide a current Tax Clearance Certificate within ten (10) calendar days, or the Division may proceed with an award to the next lowest responsive bidder, whichever is determined by the Director of Purchases to be in the best interest of the State.

Request for Quotation No.: 12882

Page 11 of 13

CERTIFICATION REGARDING

IMMIGRATION REFORM & CONTROL

All Contractors are expected to comply with the Immigration and Reform Control Act of 1986 (IRCA), as may be amended from time to time.  This Act, with certain limitations, requires the verification of the employment status of all individuals who were hired on or after November 6, 1986, by the Contractor as well as any subcontractor or sub-subcontractor.  The usual method of verification is through the Employment Verification (I-9) Form.  With the submission of this bid, the Contractor hereby certifies without exception that Contractor has complied with all federal and state laws relating to immigration and reform.  Any misrepresentation in this regard or any employment of persons not authorized to work in the United States constitutes a material breach and, at the State’s option, may subject the contract to termination and any applicable damages.

Contractor certifies that, should it be awarded a contract by the State, Contractor will comply with all applicable federal and state laws, standards, orders and regulations affecting a person’s participation and eligibility in any program or activity undertaken by the Contractor pursuant to this contract.  Contractor further certifies that it will remain in compliance throughout the term of the contract.

At the State’s request, Contractor is expected to produce to the State any documentation or other such evidence to verify Contractor’s compliance with any provision, duty, certification, or the like under the contract.

Contractor agrees to include this Certification in contracts between itself and any subcontractors in connection with the services performed under this contract.

____________________________________________	______________________

Signature, Title of Contractor	Date

Request for Quotation No.: 12882

Page 12 of 13

State of Kansas

Department of Administration

Division of Purchases

Supplier Diversity Survey Form

Why is the Division of Purchases requesting this information?

Current statutes governing the activities of the Kansas Division of Purchases do not include preferences or set-asides for Small Business Enterprises (SBEs).  The Division of Purchases is interested in determining to what extent purchase orders and contracts are awarded to SBEs under existing work efforts.  Please Note:  You must submit this form with each bid opportunity.

Persons or concerns wishing to receive a Purchase Order or Contract Award resulting from this bid opportunity must provide the information contained in this document before the award is made.  To help expedite this procurement, it is requested that you submit this form with your bid.

COMPANY DATA

Legal Company Name

Doing Business As (if applicable)

Federal Tax ID Number  / FEIN

Diversity Program Contact Name	Title

Phone Number 	Fax Number

E-Mail Address 	Company Web

Address

City 	State 	Zip Code

Legal Structure:	□ Corporation	□ Partnership	□ Non-Profit	□ Sole Proprietorship	□ LLC

Signature 	Date:

COMPANY DIVERSITY DATA

(A)	Business Classification   (See Appendix A for definitions):

Is your business a Small Business Enterprise (SBE) as defined by the SBA?

□ Yes	□ No	□ Don’t Know

Check all that Apply:	□ Disabled (DIS) 	□ SBA-Small Disadvantage Business (SDB)

□ Veteran-Owned (VBE)	□ Women-Owned (WBE)	□ Service-Disabled Veterans-Owned (DVBE)

□ African American	□ Native American	□ Minority-Owned Business Enterprise (MBE)

□ Hispanic American  	□ Asian Pacific American	□ Disadvantaged Business Enterprise (DBE)

□ Asian Subcontinent American 	□ Other: __________________________________________________________

(B)	Has your Business Classification Status been certified by a state, municipal, federal or other certifying agency?

□ No	□ Yes	Certifying Entity:

Other State of Kansas Resources for Small Business Enterprises (SBE)

Kansas Department of Commerce

Office of Minority/Women Business Development

http://www.kansas.gov/ksbdc/

Request for Quotation No.: 12882

Page 13 of 13

State of Kansas

Department of Administration

Division of Purchases

Supplier Diversity Survey Form

Appendix A

Definition of Terms

Small Business Enterprise / Concern (SBE)

SBEs are businesses that do not exceed the size standard for the product or service it is providing as measured by its employment and/or business receipts in accordance with the U.S. SBA numerical size standards.  These standards are defined as FAR 52.219-8, 13 CFR Part 121 and 13 CFR 121.410.

Disadvantaged Business Enterprise (DBE)

DBEs are defined as a business which are (a) owned by socially disadvantaged individuals who have been subjected to racial or ethnic

prejudice or cultural bias because of their identity as a member of a group without regard to their individual qualities; or (b) owned by economically disadvantaged individuals whose ability to compete in the free enterprise system has been impaired due to diminished opportunities to obtain capital and credit as compared to others in the same line of business who are not socially disadvantaged.

Disabled Business Enterprise (DIS)

DIS businesses are at least 51% owned and controlled by one or more U.S. citizens who has a physical or mental impairment which substantially limits one or more of such person’s major life activities.

Small Disadvantage Business Concern (SDB)

SDB businesses are certified by the SBA as meeting the following criteria: (1) they are small business concern and (2) must be at least 51% owned and controlled by one or more U.S. citizens who are socially and economically disadvantaged.  African Americans, Asian Pacific Americans, Asian Subcontinent Americans, Hispanic Americans and Native Americans are presumed to qualify as being socially disadvantaged.  Other individuals can qualify if they show by a preponderance of the evidence that they are socially disadvantaged.  In addition, the personal net worth of each eligible owner applicant must be less than $750,000, excluding the values of the applicant’s ownership interest in the business seeking certification and the owner’s primary residence.  Successful applicants must also meet applicable size standards for small businesses in their industry.  SDB regulations can be found in FAR 52.219-8 and 13 CFR parts 121 & 124.

Veterans-Owned Business Concern (VBE)

VBE businesses are at least 51% owned and controlled by one or more U.S. citizens who are Veterans of the U.S. Armed Forces.  In the case of any publicly owned business, at least 51% of the stock is owned by one or more veterans and one or more veterans must control the management and daily business operation.  The term “Veteran” means a person who served in the active military, naval or air service and who was discharged or released there from under conditions other than dishonorable.  VBE regulations can de found in FAR 52.219-9 & 38 USC 101 (2).

Service-Disabled Veterans-Owned Business Concern (DVBE)

DVBE businesses are at least 51% owned and controlled by one or more U.S. citizens who are service-disabled Veterans of the U.S. Armed Forces.  In the case of any publicly owned business, at least 51% of the stock is owned by one or more service-disabled veterans and one or more veterans must control the management and daily business operation.  The term “Veteran” means a person who served in the active military, naval or air service and who was discharged or released there from under conditions other than dishonorable.  The term “Service-Disabled” means a veteran of the U.S. Military Service has a service-connected disability with a disability rating of 0%-100%.  In the case of permanent or severe disability, the spouse of caregiver of such a service-disabled veteran may control the management and daily operations.  DVBE regulations can be found in FAR 52.219-9 & 38 USC 101 (2) & USC 101 (16).

Women-Owned Business Concern (WBE)

WBE businesses are at least 51% owned and controlled by one or more U.S. citizens who are female gender.  In the case of any publicly owned business, at least 51% of the stock is owned by one or more women and one or more women must control the management and daily business operations.  For Federal contracting regulations see FAR 52-219-8.

Minority-Owned Business Enterprise (MBE)

MBE businesses are at least 51% owned and controlled by one or more U.S. citizens belonging to certain ethnic minority groups.  In the case of any publicly owned business, at least 51 % of the stock is owned by one or more minorities, and one or more minorities must control the management and daily business operations.  “Ethnic Minority Groups” are people of Asian Pacific American, Asian Subcontinent American, African American, Hispanic American and Native American descent.

- African Americans: People whose origins lay in any of the Black racial groups of Africa.

- Asian Pacific Americans: People whose origins lay in Brunei, Burma, China, Guam, Indonesia, Japan, Kampuchea (Cambodia), Korea, Laos, Malaysia, Northern Mariana Islands, Republic of the Marshall Islands, Federated States of Micronesia, Republic of Palau (U.S. Trust Territory of the Pacific Islands), the Philippines, Samoa, Singapore, Taiwan, Thailand and Vietnam.

- Asian Subcontinent Americans: People whose origins lay in Bangladesh, Bhutan, India, Pakistan, Sri Lanka or Nepal.

- Hispanic Americans: People whose origins are in the South and Central America, Mexico, Puerto Rico, Cuba or the Iberian Peninsula (including Portugal).

- Native Americans: American Indians, Inuit (Eskimos), Aleuts, and native Hawaiians of Polynesian ancestry.

Date of Last Update:  March, 2007