|  |  | ### COMPLETE CABIN UTILITIES |
| --- | --- | --- |
|  |  | ### PART 1 - GENERAL |
|  |  | 1.1 SUMMARY |
| 1 | Lot | A. Project Scope: Furnish all labor, equipment, and materials necessary to complete cabin utilities as shown for Kingman State Fishing Lake, Byron Walker Wildlife Area on plans sheet. |
|  |  | All specifications for water lines, waste water system, electrical and general notes are included on the attached plan sheet. |
|  |  | Full size copies of the plan sheet may be obtained by calling: |
|  |  | Jesse Moore, Project Manager |
|  |  | 620.672.0763 |
|  |  | Sandy Veverka |
|  |  | 620.672.0766 |
|  |  | 1.2 DEFINITION |
|  |  | A. Architect/Engineer: Where the word "Architect/Engineer" is used throughout the Contract Documents, it shall be understood to mean Owner (Kansas Department of Wildlife & Parks (KDWP), Engineering Services Section). |
|  |  | 1.3 POINTS OF CONTACT |
|  |  | A. Point of Contact (P.O.C.) Names: The following individuals may be contacted regarding the Project. For technical and administrative questions concerning the Project during bidding and construction, contact: |
|  |  | Jesse Moore, Project Manager |
|  |  | Kansas Department of Wildlife and Parks |
|  |  | Engineering Section |
|  |  | 512 SE. 25th Ave. |
|  |  | Pratt, Kansas 67124 |
|  |  | 620.672.0763 |
|  |  | 1.4 SITE VISITS |
|  |  | A. Bidders Access to Site: Bidders and sub-bidders may have access to the work site by contacting the Public Lands Manager to arrange a day and time. |
|  |  | Troy Smith, Public Lands Manager |
|  |  | 8685 W. Hwy 54 |
|  |  | Cunningham, Kansas 67035 |
|  |  | 620.532.3242 |
|  |  | 1.5 AGREEMENTS FOR BIDDING |
|  |  | A. Bidder acknowledges and agrees to submit his Bid in accordance with the provisions required by the Bid Form (State of Kansas Request for Quotation) supplied to the Bidder by Kansas Division of Purchases and to the following terms and conditions for bidding. Time and place for submittal of Bids is indicated on the Bid Form. |
|  |  | B. The Project shall be constructed under a single Contract and will be awarded to the responsible Bidder offering the lowest bid price and meeting the requirements for bidding and requirements of the Contract Documents. The lowest bid price will be calculated by the sum of the Base Bid and the total of any Alternates that are accepted by Owner after all Bids are opened. |
|  |  | C. Failure to acknowledge receipt of Addenda issued may be cause for Owner's rejection of a Bid. |
|  |  | D. An incomplete Bid or information included but not solicited on the Bid Form may be cause for Owner's rejection of a bid. |
|  |  | E. A Bid may be considered incomplete or non-responsive if it fails to quote a price for an Alternate or other separate item identified and solicited on the Bid Form supplied to the Bidder. |
|  |  | F. The Kansas Division of Purchases reserves the right to reject any or all Bids and to waive technicalities and formalities for the acceptance or rejection of Bids, should such action be deemed in the best interest of the Owner. |
|  |  | G. The Owner reserves the right to accept or reject any or all Alternates submitted with the Bid before award of the Contract. |
|  |  | H. In compliance with K.S.A. 75-3741, subcontractors which the Contractor proposes to employ for the work shall be listed. |
|  |  | I. It is the responsibility of the Contractor to supply his prospective subcontractors with copies of the Contract Documents for bidding. Neither the Owner nor the Kansas Division of Purchases will be responsible for supplying prospective subcontractors with Contract Documents for bidding. |
|  |  | 1.6 ADDENDA |
|  |  | A. Acknowledgment of Receipt of Addenda: I/we __________________________ have received and considered the following Addenda in the submitted Bid for this Project. (Bidder shall fill in signature above using same person as signing the Bid Form and shall initial each applicable Addenda space below.) |
|  |  | None (_____) #1 (_____) #2 (_____) #3 (_____) #4 (_____) #5 (_____) |
|  |  | B. Substantiation: Bidder shall attach a copy of this completed page to the Bid Form. |
|  |  | 1.7 TIME OF COMPLETION |
|  |  | A. Calendar Days: Contractor agrees, if awarded the Contract, to complete the Work of the Contract within 30 calendar days from the date of the "Notice to Proceed" issued by the KDWP project manager. |
|  |  | B. Substantial Completion is defined as: All products furnished, installed, and work completed in accordance with the contract documents with exception of deficient items noted on final inspection. |
|  |  | 1.8 LIQUIDATED DAMAGES |
|  |  | A. Amount: Liquidated damages for the Project are established at the rate of $100 (One Hundred) dollars per calendar day. |
|  |  | 1.9 FEDERAL TAX |
|  |  | A. Federal Tax Included: Bidder acknowledges that the cost for the Federal Excise Tax has been included in quoted prices on the Bid Form, to cover the cost of said tax on all items of construction, equipment, and material subject to such tax. |
|  |  | 1.10 PRE-CONSTRUCTION |
|  |  | A. Requirements: Prior to starting construction the following items shall be completed. |
|  |  | 1. Contractor's completion of a Pre-Construction Meeting with KDWP personnel. |
|  |  | 2. Receipt of "Purchase Order" from State of Kansas, Division of Purchases. |
|  |  | 3. Receipt of "Notice to Proceed" from Kansas Department of Wildlife & Parks, Engineering Services Section. |
|  |  | 1.11 SUBMITTAL |
|  |  | A. Addressing of Submittals: Submittals, notifications, and reports required by the Specifications and Conditions of the Contract shall be sent and addressed to the following, unless otherwise indicated in the Contract Documents. |
|  |  | Jesse Moore, Project Manager |
|  |  | Kansas Department of Wildlife and Parks |
|  |  | Engineering Section |
|  |  | 512 SE. 25th Ave. |
|  |  | Pratt, Kansas 67124 |
|  |  | 620.672.0763 |
|  |  | B. Format and Identification: Project number shall be on all submittals. Submittals shall be approved by the General Contractor prior to submittal to KDWP. All copies shall have an indication of General Contractor approval. |
|  |  | C. Marking and Legibility: Submittals in the form of manufacturer's standard preprinted data and brochures shall be suitably marked by Contractor to designate the information, materials, and products applicable to the Project. Such markings shall be reproducible by means of standard photocopy machinery. Color highlighting as a means of marking is not acceptable. |
|  |  | D. Copies: Contractor shall provide 2 copies of each submittal for Owner's records plus the number of copies needed by Contractor. |
|  |  | 1.12 GUARANTEES AND WARRANTIES |
|  |  | A. General: Provide guaranties and warranties in accordance with the General Conditions of the Contract and as required by the Specifications. |
|  |  | B. Submittal: Warranties and guarantees shall be addressed and submitted to: |
|  |  | Troy Smith, Public Lands Manager |
|  |  | 8685 W. Hwy 54 |
|  |  | Cunningham, Kansas 67035 |
|  |  | 620.532.3242 |
|  |  | 1.13 MATERIAL, EQUIPMENT, AND SUBSTITUTIONS |
|  |  | A. Materials and Products: In general, the Contract Documents identify the required materials and equipment by naming one or more manufacturer's brand name, model number, catalog number, or other similar proprietary identification. Where one or more proprietary identifications or manufacturers is listed for a category of material and equipment, the intent is that the Contractor may provide any of those listed. Bids shall be based on Contractor providing only those materials and equipment named and identified in the Contract Documents. |
|  |  | B. Material Quality: Contractor provided material and equipment shall be new, unused, and undamaged when delivered to the site. |
|  |  | C. Substitutions: Materials and equipment not named in the Contract Documents which the Contractor wishes to provide will be considered as a substitution in accordance with the following procedures and conditions and may be provided only when approved by the Owner. |
|  |  | 1. Substitution Requests: When request for substitution is made after execution of the Contract, Contractor shall request substitution in writing and such request shall be accompanied by complete information to allow Owner's analysis including, manufacturer's and product identification, technical data including performance, dimensional and operational characteristics, cost data impact on time for completion, samples, and other information necessary for analysis. Include an itemized comparison of the qualities and characteristics of the specified item versus those of the substitution item. Submit 2 copies of request and accompanying information and clearly identify as "Request for Substitution" followed by appropriate Project Number. |
|  |  | 2. Conditions: Owner will consider substitutions only under one or more of the following conditions. |
|  |  | a. Required for compliance with subsequent interpretations of code requirements. |
|  |  | - Unavailability of a specified item, through no fault of Contractor. |
|  |  | c. Subsequent information discloses inability of specified item to perform properly or fit in designated space. |
|  |  | d. Manufacturer of fabricator refuses to certify, guarantee, or warranty performance of specified item. |
|  |  | e. When the Owner determines that in his judgment a substitution would be substantially to the Owner's best interest in terms of cost, time or other considerations. |
|  |  | 3. Contractor's Representations: By submitting a request for substitution, Contractor represents the following: |
|  |  | a. Proposed substitution has been personally investigated by Contractor. |
|  |  | - The substitution material or equipment carries the same or better warranty or guarantee as the specified item. |
|  |  | c. Cost data presented is complete and includes all related costs under the Contract, and Contractor waives all claims for additional costs related to the substitution and the Project which subsequently become apparent. |
|  |  | - Installation of an approved substitution shall be coordinated with all portions of the Work, and that changes as may be necessary shall be made so that the Work is complete in all respects. |
|  |  | 1.14 TEMPORARY FACILITIES AND CONTROLS |
|  |  | A. General: Contractor shall provide temporary facilities and controls necessary to complete the Work. Remove Contractor provided temporary facilities and controls upon completion of the Work and sooner when no longer needed. |
|  |  | B. Contractor's Field Office and Storage: Space for a field office within the work area is acceptable. An exterior area to accommodate a field office or material storage at the work site, if requested by Contractor, will be provided at a location to be designated by the Owner. Owner is not responsible for safe keeping or protection of materials of equipment stored on Owner's property. |
|  |  | C. Toilet Facilities: Contractor must supply own facilities. |
|  |  | D. Water: Contractor may use Owner's existing sources. |
|  |  | E. Electric and Lighting: Contractor must supply for own use. |
|  |  | F. Telephone: Not provided. |
|  |  | 1.15 CODES AND REGULATIONS |
|  |  | A. General: Comply with the following Codes and Regulations of the date or edition indicated, or if not indicated, of the most current date or edition published. The following list is not intended to be all inclusive of the Codes or Regulations to which the Project is subject to. |
|  |  | 1. Uniform Building Code, 1997. |
|  |  | 1.16 ADDITIONAL REQUIREMENTS |
|  |  | A. Existing Facilities: The Contractor shall take all necessary precautions to insure against damage to existing facilities and their contents. Any damaged items shall be repaired or replaced by the Contractor at no additional cost to the Owner. |
|  |  | B. Protection of Work: The Contractor shall take all necessary precautions to insure against damage to Products and Work. Any damaged items shall be replaced, or repaired, so as to eliminate evidence of repair, at no additional cost to the Owner. |
|  |  | C. Project Maintenance: Remove debris and rubbish from the work site as frequently as necessary to avoid safety hazards and unsightliness, and at the end of each workday |
|  |  | D. Service Interruption: The Contractor shall take all necessary precautions to maintain sewage service to existing users, make all necessary contacts with users for interruption of service and limit the interruptions as much as possible. |
|  |  | 1.17 INSPECTIONS |
|  |  | A. Inspections: Inspections will be conducted by KDWP Engineering Services Section personnel as frequently as deemed necessary. |
|  |  | B. Notifications: The Contractor shall give KDWP personnel at the Engineering Services Section office a minimum of 24 (twenty-four) hours notice prior to any concrete pours or concealment or covering for inspection purposes. |
|  |  | C. An inspection conducted by Engineering Services Section personnel does not relieve Contractor from compliance of Contract Documents. |
|  |  | TOTAL: $_____________ |  |
|  |  | Bid results will not be given over the phone. Results can be obtained by attending the public bid opening or by sending $3.00 with the bid or mail your request, include with either a self addressed stamped envelope to the Kansas Division of Purchases. Attn: Bid Results/Copies 900 SW Jackson Room 102-N Topeka, Ks. 66612. Please reference quote number on check. |
|  |  | SALES TAX DETERMINATION: |
|  |  | 1. This Project has been determined by the Kansas Department of Revenue to be subject to Kansas State Sales Tax. The cost of said tax must be INCLUDED in all Bid and Contract prices. Sales tax includes all applicable state, county and city sales tax. |
|  |  | 2. A general contractor or other contractor who contracts directly with the State of Kansas or one of its agencies shall pay tax as follows: |
|  |  | a. Labor: No tax will be assessed since the labor is purchased directly by the State or one of its agencies. |
|  |  | - Materials: Taxes will be assessed on material purchased for the project. |
|  |  | 3. A subcontractor on this project shall pay tax as follows: |
|  |  | a. Labor: Taxes will be assessed because this labor is not being purchased directly by the state or one of its agencies. (However, labor will not be taxed on projects calling for the original construction of a building or for repair or remodeling of a residence. |
|  |  | - Materials: Taxes will be assessed on materials purchased for the project. |
|  |  | 4. Whether sales tax should be paid to the Department of Revenue by a subcontractor on the subcontractor's labor services will depend upon whether the project qualifies as the original construction of a building or facility or as the repair or remodeling of a residence. The contractor and each subcontractor will be responsible for determining whether labor services for the project qualifies for such exemption and for including the correct amount of state, county and city sales tax applicable to this project. Contractors and subcontractors must include all sales tax due on materials, which are not exempt on this project. Any questions should be addressed to the Kansas Department of Revenue, Division of Taxation, Taxpayer Assistance Center, (785) 296-0222. |
|  |  | 5. A general contractor or other contractor that contracts directly with the State of Kansas shall obtain copies of the Kansas Retail Sales Tax Registration Certificates from all it's subcontractors on this project and have them available at the jobsite upon request. |
|  |  | Accounts Receivable Set-Off Program: During the course of this contract if the vendor is found to owe a debt to the State of Kansas, agency payments to the vendor may be intercepted / setoff by the State of Kansas Accounts Receivable Setoff Program. Notice of the setoff action will be provided to the vendor. The vendor shall credit the account of the agency making the payment in an amount equal to the funds intercepted. |
|  |  | K.S.A. 75-6201 et seq. allows the Director of Accounts & Reports to setoff funds the State of Kansas owes vendors against debts owed by the vendors to the State of Kansas. Payments setoff in this manner constitute lawful payment for services or goods received. The vendor benefits fully from the payment because its' obligation to the State is reduced by the amount subject to setoff. |
|  |  | #### CRIMINAL OR CIVIL OFFENSE OF AN INDIVIDUAL OR ENTITY THAT CONTROLS A COMPANY OR ORGANIZATION OR WILL PERFORM WORK UNDER THIS CONTRACT: Any conviction for a criminal or civil offense that indicates a lack of business integrity or business honesty must be disclosed. This includes (1) conviction of a criminal offense as an incident to obtaining or attempting to obtain a public or private contract or subcontract or in the performance of such contract or subcontract; (2) conviction under state or federal statutes of embezzlement, theft, forgery, bribery, falsification or destruction of records, receiving stolen property; (3) conviction under state or federal antitrust statutes; and (4) any other offense to be so serious and compelling as to affect responsibility as a state contractor. For the purpose of this section, an individual or entity shall be presumed to have control of a company or organization if the individual or entity directly or indirectly, or acting in concert with one or more individuals or entities, owns or controls 25 percent or more of its equity, or otherwise controls its management or policies. Failure to disclose an offense may result in disqualification of the bid or termination of the contract. |
|  |  | ### TAX CLEARANCE INSTRUCTIONS |
|  |  | PLEASE NOTE: THIS INFORMATION HAS CHANGED EFFECTIVE |
|  |  | 10/27/09 |
|  |  | A "Tax Clearance" is a comprehensive tax account review to determine and ensure that the account is compliant with all primary Kansas Tax Laws administered by the Kansas Department of Revenue (KDOR) Director of Taxation. Information pertaining to a Tax Clearance is subject to change(s), which may arise as a result of a State Tax Audit, Federal Revenue Agent Report, or other lawful adjustment(s). |
|  |  | To obtain a Tax Clearance Certificate, you must: |
|  |  | - Go to http://www.ksrevenue.org/taxclearance.htm to request a |
|  |  | Tax Clearance Certificate |
|  |  | - Return to the website the following working day to see if |
|  |  | KDOR will issue the certificate |
|  |  | - If issued an official certificate, print it and attach it to your bid |
|  |  | response |
|  |  | - If denied a certificate, engage KDOR in a discussion about why |
|  |  | a certificate wasn't issued |
|  |  | Bidders (and their subcontractors) are expected to submit a current Tax Clearance Certificate with every event response. |
|  |  | Per KSA 75-3740-(c), the Director of Purchases may reject the bid of any bidder who is in arrears on taxes due the State of Kansas. The Secretary of the Kansas Department of Revenue is authorized to exchange such information with the Director of Purchases as is necessary to determine a bidder's tax clearance status, notwithstanding any other provision of law prohibiting disclosure of the contents of taxpayer records or information. |
|  |  | Please Note: Individual and business applications are available. For |
|  |  | applications entered prior to 5:00 PM Monday through Friday, results |
|  |  | typically will be available the following business day. Tax clearance |
|  |  | requests may be denied if the request includes incomplete or incorrect |
|  |  | information. |
|  |  | Please Note: You will need to sign back into the KDOR website to view and print the official tax clearance certificate. |
|  |  | Information about Tax Registration can be found at the following website: |
|  |  | http://www.ksrevenue.org/busregistration.htm |
|  |  | The Division of Purchases reserves the right to confirm tax status of all potential contractors and subcontractors prior to the release of a purchase order or contract award. |
|  |  | In the event that a current tax certificate is unavailable, the Division of Purchases reserves the right to notify a bidder (one that has submitted a timely event response) that they have to provide a current Tax Clearance Certificate within ten (10) calendar days, or the Division may proceed With an award to the next lowest responsive bidder, whichever is determined by the Director of Purchases to be in the best interest of the State. |

Quotation No.: 12898

Page 11 of 13

CERTIFICATION REGARDING

IMMIGRATION REFORM & CONTROL

All Contractors are expected to comply with the Immigration and Reform Control Act of 1986 (IRCA), as may be amended from time to time.  This Act, with certain limitations, requires the verification of the employment status of all individuals who were hired on or after November 6, 1986, by the Contractor as well as any subcontractor or sub-subcontractor.  The usual method of verification is through the Employment Verification (I-9) Form.  With the submission of this bid, the Contractor hereby certifies without exception that Contractor has complied with all federal and state laws relating to immigration and reform.  Any misrepresentation in this regard or any employment of persons not authorized to work in the United States constitutes a material breach and, at the State’s option, may subject the contract to termination and any applicable damages.

Contractor certifies that, should it be awarded a contract by the State, Contractor will comply with all applicable federal and state laws, standards, orders and regulations affecting a person’s participation and eligibility in any program or activity undertaken by the Contractor pursuant to this contract.  Contractor further certifies that it will remain in compliance throughout the term of the contract.

At the State’s request, Contractor is expected to produce to the State any documentation or other such evidence to verify Contractor’s compliance with any provision, duty, certification, or the like under the contract.

Contractor agrees to include this Certification in contracts between itself and any subcontractors in connection with the services performed under this contract.

__________________________________________________________		___________________________

Signature, Title of Contractor	Date

Quotation No.: 12898

Page 12 of 13

State of Kansas

Department of Administration

Division of Purchases

Supplier Diversity Survey Form

Why is the Division of Purchases requesting this information?

Current statutes governing the activities of the Kansas Division of Purchases do not include preferences or set-asides for Small Business Enterprises (SBEs).  The Division of Purchases is interested in determining to what extent purchase orders and contracts are awarded to SBEs under existing work efforts.  Please Note:  You must submit this form with each bid opportunity.

Persons or concerns wishing to receive a Purchase Order or Contract Award resulting from this bid opportunity must provide the information contained in this document before the award is made.  To help expedite this procurement, it is requested that you submit this form with your bid.

COMPANY DATA

Legal Company Name

Doing Business As (if applicable)

Federal Tax ID Number  / FEIN

Diversity Program Contact Name	Title

Phone Number 	Fax Number

E-Mail Address 	Company Web

Address

City 	State 	Zip Code

Legal Structure:	□ Corporation	□ Partnership	□ Non-Profit	□ Sole Proprietorship	□ LLC

Signature 	Date:

COMPANY DIVERSITY DATA

(A)	Business Classification   (See Appendix A for definitions):

Is your business a Small Business Enterprise (SBE) as defined by the SBA?

□ Yes	□ No	□ Don’t Know

Check all that Apply:	□ Disabled (DIS) 	□ SBA-Small Disadvantage Business (SDB)

□ Veteran-Owned (VBE)	□ Women-Owned (WBE)	□ Service-Disabled Veterans-Owned (DVBE)

□ African American	□ Native American	□ Minority-Owned Business Enterprise (MBE)

□ Hispanic American  	□ Asian Pacific American	□ Disadvantaged Business Enterprise (DBE)

□ Asian Subcontinent American 	□ Other: __________________________________________________________

(B)	Has your Business Classification Status been certified by a state, municipal, federal or other certifying agency?

□ No	□ Yes	Certifying Entity:

Other State of Kansas Resources for Small Business Enterprises (SBE)

Kansas Department of Commerce

Office of Minority/Women Business Development

http://www.kansas.gov/ksbdc/

Quotation No.: 12898

Page 13 of 13

State of Kansas

Department of Administration

Division of Purchases

Supplier Diversity Survey Form

Appendix A

Definition of Terms

Small Business Enterprise / Concern (SBE)

SBEs are businesses that do not exceed the size standard for the product or service it is providing as measured by its employment and/or business receipts in accordance with the U.S. SBA numerical size standards.  These standards are defined as FAR 52.219-8, 13 CFR Part 121 and 13 CFR 121.410.

Disadvantaged Business Enterprise (DBE)

DBEs are defined as a business which are (a) owned by socially disadvantaged individuals who have been subjected to racial or ethnic

prejudice or cultural bias because of their identity as a member of a group without regard to their individual qualities; or (b) owned by economically disadvantaged individuals whose ability to compete in the free enterprise system has been impaired due to diminished opportunities to obtain capital and credit as compared to others in the same line of business who are not socially disadvantaged.

Disabled Business Enterprise (DIS)

DIS businesses are at least 51% owned and controlled by one or more U.S. citizens who has a physical or mental impairment which substantially limits one or more of such person’s major life activities.

Small Disadvantage Business Concern (SDB)

SDB businesses are certified by the SBA as meeting the following criteria: (1) they are small business concern and (2) must be at least 51% owned and controlled by one or more U.S. citizens who are socially and economically disadvantaged.  African Americans, Asian Pacific Americans, Asian Subcontinent Americans, Hispanic Americans and Native Americans are presumed to qualify as being socially disadvantaged.  Other individuals can qualify if they show by a preponderance of the evidence that they are socially disadvantaged.  In addition, the personal net worth of each eligible owner applicant must be less than $750,000, excluding the values of the applicant’s ownership interest in the business seeking certification and the owner’s primary residence.  Successful applicants must also meet applicable size standards for small businesses in their industry.  SDB regulations can be found in FAR 52.219-8 and 13 CFR parts 121 & 124.

Veterans-Owned Business Concern (VBE)

VBE businesses are at least 51% owned and controlled by one or more U.S. citizens who are Veterans of the U.S. Armed Forces.  In the case of any publicly owned business, at least 51% of the stock is owned by one or more veterans and one or more veterans must control the management and daily business operation.  The term “Veteran” means a person who served in the active military, naval or air service and who was discharged or released there from under conditions other than dishonorable.  VBE regulations can de found in FAR 52.219-9 & 38 USC 101 (2).

Service-Disabled Veterans-Owned Business Concern (DVBE)

DVBE businesses are at least 51% owned and controlled by one or more U.S. citizens who are service-disabled Veterans of the U.S. Armed Forces.  In the case of any publicly owned business, at least 51% of the stock is owned by one or more service-disabled veterans and one or more veterans must control the management and daily business operation.  The term “Veteran” means a person who served in the active military, naval or air service and who was discharged or released there from under conditions other than dishonorable.  The term “Service-Disabled” means a veteran of the U.S. Military Service has a service-connected disability with a disability rating of 0%-100%.  In the case of permanent or severe disability, the spouse of caregiver of such a service-disabled veteran may control the management and daily operations.  DVBE regulations can be found in FAR 52.219-9 & 38 USC 101 (2) & USC 101 (16).

Women-Owned Business Concern (WBE)

WBE businesses are at least 51% owned and controlled by one or more U.S. citizens who are female gender.  In the case of any publicly owned business, at least 51% of the stock is owned by one or more women and one or more women must control the management and daily business operations.  For Federal contracting regulations see FAR 52-219-8.

Minority-Owned Business Enterprise (MBE)

MBE businesses are at least 51% owned and controlled by one or more U.S. citizens belonging to certain ethnic minority groups.  In the case of any publicly owned business, at least 51 % of the stock is owned by one or more minorities, and one or more minorities must control the management and daily business operations.  “Ethnic Minority Groups” are people of Asian Pacific American, Asian Subcontinent American, African American, Hispanic American and Native American descent.

- African Americans: People whose origins lay in any of the Black racial groups of Africa.

- Asian Pacific Americans: People whose origins lay in Brunei, Burma, China, Guam, Indonesia, Japan, Kampuchea (Cambodia), Korea, Laos, Malaysia, Northern Mariana Islands, Republic of the Marshall Islands, Federated States of Micronesia, Republic of Palau (U.S. Trust Territory of the Pacific Islands), the Philippines, Samoa, Singapore, Taiwan, Thailand and Vietnam.

- Asian Subcontinent Americans: People whose origins lay in Bangladesh, Bhutan, India, Pakistan, Sri Lanka or Nepal.

- Hispanic Americans: People whose origins are in the South and Central America, Mexico, Puerto Rico, Cuba or the Iberian Peninsula (including Portugal).

- Native Americans: American Indians, Inuit (Eskimos), Aleuts, and native Hawaiians of Polynesian ancestry.

Date of Last Update:  March, 2007