REQUEST FOR PROPOSAL

STREAM GAUGING SERVICES

RFP02229

SECTION 1 - GENERAL INFORMATION

Purpose

The Division of Purchasing is seeking bids for the Idaho Department of Water Resources.  The purpose of this request for proposal is to provide applicants with sufficient information concerning stream gauging services required by the state of Idaho, Department of Water Resources (hereinafter referred to as “IDWR”) to enable them to prepare and to submit proposals for consideration.

The objective of the services required is to establish and maintain 17 stream gauges in Idaho. The vendor will own, operate and maintain stream flow stage monitoring equipment, or equivalent, with continuous recorders from January 1, 2010 to December 31, 2010.

Right of Rejection

IDWR reserves the right to reject any or all proposals received as a result of this request.  Award will be made based on the evaluations of the proposals resulting from this request.

IDWR is not liable for any costs incurred by firms prior to issuance of an agreement and notice to proceed.

Timeline:

Bidders Conference                                             October 23, 2009

Questions Due		 	                   October 30, 2009

Questions Answered (approximate) 		     November 6, 2009

Closing Date				     November 25, 2009

Issuing Office and Submission of Questions

This Request for Proposal is issued by the Division of Purchasing.  The Division of Purchasing is the only contact for this solicitation.

All correspondence will be in writing.  In the event that it becomes necessary to revise any part of this RFP, addenda will be posted at www.sicomm.net. It will be the responsibility of the Proposer to monitor www.sicomm.net for any updates or amendments.  Any oral interpretations or clarifications of this RFP shall not be relied upon.   All questions must be received by no later than October 30, 2009 at 5PM Mountain Time to be considered.

## Pre-proposal Conference

A pre-proposal conference will occur on October 23, 2009 at 9am at the Idaho Department of Water Resources, 322 East Front St., Boise, ID. To participate by phone call (713) 577-1201 and enter Participant Code:  260503. It is highly recommended for potential proposers to attend in person.

How to Submit Proposals

Proposals are to be submitted manually and must be received at the office of the Division of Purchasing and

time stamped using the Division’s time stamp, no later than the date and time set forth for the Bid Closing.

Do not fax or e-mail your response.

Proposals must be sealed, and the outer package clearly labeled to identify the solicitation.  One (1) original

and four (4) copies of the Proposal, marked “Technical Proposal - Original” along with one (1) electronic

copy (on CD or USB).  One (1) original of the Proposer’s cost proposal marked “Cost Proposal - Original”

along with one (1) copy. The cost proposal and copy are to be sealed separate from the technical proposal

and marked confidential “Cost Proposal” on the envelope.

Proposals must include a completed State of Idaho Signature Page, with an original signature.  Failure to include a signed, complete original State of Idaho Signature Page will result in a finding that the Proposal is non-responsive, and no further consideration will be given to the Proposal. If you have a web site, please write it on the State of Idaho Signature Page.

Proposals should provide straight forward and concise assurances of the vendor’s ability to satisfy the requirements of the Proposal and must be adequate for evaluation.  Cost must be broken down in the Bid by station, Attachment A – Stream Gauging Cost Sheet must be submitted.

Administration of Contract

- Funding:  The project is funded by a Pacific Coast Salmon Recovery Fund 2008 grant (Office of Species Conservation Project Number 006-08-SA-B4).  IDWR will administer the project according to the terms and conditions of the award(s) and state laws and guidelines.

- Method of Payment:  The vendor shall invoice the Department quarterly for all work and services performed under this Contract. The payment request is to be submitted in a form prescribed by the State.

- Terms:  Services requested by this RFP are for the calendar year January 1, 2010 to December 31, 2010, with installation of gauges no later than February 1, 2010.  Contract period will be for one year with the option of up to four one-year renewals.  Renewal of the contract(s) will depend upon satisfactory stream flow data.  Price increases for succeeding renewal years shall not exceed the consumer price index for urban wage earners and clerical workers in “West Urban” as published by the Bureau of Labor Statistics effective February 1 proceeding the next fiscal year, with a maximum adjustment in any year of five (5) percent.

Delivery Requirements

The contract vendor will provide to IDWR an annual report at the end of each water year.  The report will include final data for each station calculated from periodic discharge measurements taken throughout the year.  The annual report will be due no later than March 1 following the end of the calendar year.  Data will also be posted on the Internet.  Fifteen minute data will be available during the irrigation season to IDWR by request as needed.

SECTION 2 - SERVICE REQUIREMENTS

General

Vendors must submit proposals for installation, stream gauging, and operation and maintenance.  All service requirements for each service type listed below are considered mandatory and must be provided by the vendor.

# Station Locations

The gauging stations shall be located at or near the existing station sites are as follows:

- Agency Creek above diversions (13305020) Sec. 26, T. 19N, R. 24E, Lemhi County.

- Big Eightmile Creek above diversions (13304495) Sec. 07, T. 15N, R. 25E, Lemhi County

- Big Springs Creek near Leadore (13304100) Sec. 18, T. 16N, R. 26E, Lemhi County.

- Big Springs Creek near the Lemhi River confluence (13304250) Sec. 12, T. 16N, R. 25E, Lemhi County.

- Big Timber Creek above diversions (13303500) Sec. 20, T. 15N, R. 26E, Lemhi County.

- Bohannon Creek near Baker (13305300) Sec. 28, T. 21N, R. 23E, Lemhi County.

- Carmen Creek near the Salmon River confluence (13305650), Sec. 08, T. 22N, R. 22E, Lemhi County.

- Carmen Creek above diversions (13305640) Sec. 07, T. 23N, R. 23E, Lemhi County

- Challis Creek below diversions, near Hwy 93 Bridge (13305650) Sec. 08, T. 22N, R. 22E, Custer County.

- Challis Creek at existing USGS gage site (13299210) Sec. 05, T. 14N., R. 18E., Custer County

- Hayden Creek near Lemhi River confluence, Sec. 28, T. 18N, R. 24E, Lemhi County

- Lemhi River above Big Springs confluence (13304160) Sec. 12, T. 16N, R. 25E, Lemhi County

- Lemhi River at Cottom Lane (13304400) Sec. 03, T. 16N, R. 25E, Lemhi County

- Lemhi River above L63 near Leadore (13303069) Sec. 27, T. 16N, R. 26E, Lemhi County

- Little Springs Lower near Lemhi River confluence (13304450) Sec. 25, T. 17N, R. 24E, Lemhi County

- Patterson-Big Springs near May (13301900) Sec. 26, T. 15N, R. 21E, Lemhi County

- Pahsimeroi River above P-9 diversion (13301895) Sec. 27, T. 15N, R. 21E,, Lemhi County

Equipment and Maintenance

If appropriate, vendors must obtain easements from the appropriate agencies and private landowners for each gauging station.

Currently gauges are operated by Idaho Power Company and Water District 74.  If more information is needed about the gauging locations see the contact information for each location below.  Agency Creek above diversions, Big Eightmile Creek above diversions, Big Springs Creek near Leadore, Big Springs Creek near the Lemhi River confluence, Big Timber Creek above diversions, Bohannon Creek near Baker, Carmen Creek near the Salmon River, Carmen Creek above diversions, Challis Creek below diversions near Hwy 93 Bridge, Challis Creek as existing USGS gage site, Lemhi river above Big Springs confluence, Lemhi River at Cottom Lane, Lemhi River above L63, Little Springs near Lemhi River confluence, Patterson-Big Springs near May, and the Pahsimeroi River above the P-9 diversion have equipment owned by Idaho Power Company.  The contact persons are Mike Campbell (388-2473) and Peter Vidmar (388-2939).  Hayden Creek near the Lemhi River confluence has equipment owned by Water District 74.  The contact person for this gage is the Water Master Rick Sager (303-7449).

The Vendor will make routine discharge measurements as outlined in the table below.  The Vendor will calculate shifts and new rating curves if appropriate.

Table 1: Discharge Measurement Schedule

| Station | Measurement Schedule |
| --- | --- |
| Agency Creek above diversions | No less than every six weeks during irrigation season (March 15 – Nov 1) |
| Big Eightmile Creek above diversions | No less than every six weeks during irrigation season (March 15 – Nov 1) |
| Big Timber Creek above diversions | No less than every six weeks during irrigation season (March 15 – Nov 1) |
| Carmen Creek above diversions | No less than every six weeks during irrigation season (March 15 – Nov 1) |
| Challis Creek at existing USGS gage site | No less than every six weeks during irrigation season (March 15 – Nov 1) |
| Little Springs Creek Lower | No less than every six weeks during irrigation season (March 15 – Nov 1) |
| Big Springs Creek near Leadore | No less than every six weeks. |
| Big Springs Creek near the Lemhi River confluence. | No less than every six weeks. |
| Bohannon Creek near Baker | No less than every six weeks. |
| Carmen Creek near the Salmon River confluence | No less than every six weeks. |
| Challis Creek below diversions, near Hwy 93 Bridge | No less than every six weeks. |
| Hayden Creek near Lemhi River confluence | No less than every six weeks. |
| Lemhi River above Big Springs confluence | No less than every six weeks. |
| Lemhi River at Cottom Lane | No less than every six weeks. |
| Lemhi River above L63 near Leadore | No less than every six weeks. |
| Patterson-Big Springs near May | No less than every six weeks. |
| Pahsimeroi River above P-9 diversion | No less than every six weeks. |

The vendor will supply and maintain all equipment.  All equipment will remain the property of the vendor.  The vendor is responsible for repair and routine maintenance of the stream gauging equipment.   IDWR does not have discretionary funds available for repair, maintenance or loss of equipment due to vandalism.  Proposals must discuss how vendor will address equipment breakage and subsequent repairs (i.e., replace, swap out equipment, remove & repair, etc.).  IDWR would expect equipment to be replaced as quickly as possible to ensure data continuity.  An assessment of the equipment will be reported to IDWR’s project coordinator within three working days of any problem.  Equipment will be repaired within five working days unless an alternative agreement is reached with IDWR.

Data Collection

Vendor must operate gauging stations and equipment to collect and store flow measurements at 15-minute intervals.  The accuracy of stream flow data depends on the stability of the stage-discharge relation and the accuracy of stage and discharge measurements.  Vendor will discuss how data will be analyzed to report the best possible daily discharge.  Vendor must discuss methods and equipment to be used to obtain discharge measurements at each site.

Vendor must discuss quality assurance procedures to be followed to ensure the quality and reliability of the surface water data.  Tolerance limits for discharge measurements must meet or exceed USGS standards as described in, “Techniques of Water-Resources Investigations of the United States Geological Survey, Book 3”.  A discussion of frequency and method of calibrating equipment must be included in the proposal.  Vendor must state how often gauging station level surveys will be conducted.

The vendor is required to make and maintain a field log of routine field activities such as discharge measurements and for reporting unusual conditions encountered in the field.  Field notes should identify procedures followed in the field, describe any deviations from standard practice, and report any information that could affect the use, analysis or interpretation of the data.  Field notes should be recorded in an acceptable format as a permanent record.  Notes must be neat, readable, and leave no doubt about interpretation.  All information should be recorded as collected.

The vendor is required to make the average daily data available to the public via the internet on a web site maintained by the vendor.   The vendor is required to transmit quarterly a data file in a mutually agreed upon electronic format containing the 15-minute data and average daily data collected for each site.

Up to ten additional measurements may be requested by the Department.  Vendor will be expected to visit the site within five working days of the request for a discharge or indirect measurement.  Vendor will include cost of extra measurements for each gauging location in their proposal.

SECTION 3 - SELECTION PROCEDURE

Evaluation Criteria

Proposals will be scored by an evaluation committee consisting of IDWR staff and other qualified personnel, using the criteria listed below.  Each proposer must provide two references for the Department to contact and to evaluate the ability of the vendor to provide the necessary services. Please submit Company Name, Contact Person’s Name and Phone number for each reference. Proposals must satisfy all requirements set forth in the RFP.

- Gauging capability:  450 points

- Completeness: 150 Points

- References:  100 points.

- Cost** 300 points.

**	The qualified proposer submitting the lowest price will receive the maximum number of points (300).  Other proposers will receive an appropriate number of points as determined by the following formula:

N = (LQ/Q) * (maximum points)

Where:

N = number of cost category points awarded

LQ = lowest quote

Q = proposer’s cost quote

SECTION 4 – AWARD

The contract will be awarded to the responsive, responsible Proposer with the highest total number of points.  The State of Idaho reserves the right to reject any or all proposals received as a result of this request.   Awards will only be made on the basis of proposals resulting from this request.

RFP02229

Attachment A

Stream Gauging Services Cost Sheet Analysis

| Gauging Stations | Cost |
| --- | --- |
| Agency Creek above diversions |  |
| Big Eightmile Creek above diversions |  |
| Big Timber Creek above diversions |  |
| Carmen Creek above diversions |  |
| Challis Creek at existing USGS gage site |  |
| Little Springs Creek Lower |  |
| Big Springs Creek near Leadore |  |
| Big Springs Creek near the Lemhi River confluence. |  |
| Bohannon Creek near Baker |  |
| Carmen Creek near the Salmon River confluence |  |
| Challis Creek below diversions, near Hwy 93 Bridge |  |
| Hayden Creek near Lemhi River confluence |  |
| Lemhi River above Big Springs confluence |  |
| Lemhi River at Cottom Lane |  |
| Lemhi River above L63 near Leadore |  |
| Patterson-Big Springs near May |  |
| Pahsimeroi River above P-9 diversion |  |